## Your Design

Thank you for having us in your home. Below is a summary of your proposed design.

HOUSE LOSS

9.99

kW

ROOMS

19

Total

EXT. TEMP.

-2.0

oC

## Second Floor

### Loft room

- Heat loss: 0.47 kW

- Design temperature: 21.0 oC

- Radiators

- Existing - type 11 - 700 x 800 mm

- 880 W@dT50

### Boiler room

- Heat loss: 0.17 kW

- Design temperature: 21.0 oC

- Radiators

- Repurposed - type 22 - 400 x 1000 mm

- 1140 W@dT50

## Ground Floor

### Study

- Heat loss: 0.44 kW

- Design temperature: 21.0 oC

- Underfloor heating

- New - Low Profile Overlay

- Output 794 W @ 27.0 oC floor temp

### Entrance porch

- Heat loss: 0.30 kW

- Design temperature: 21.0 oC

- Radiators

- Repurposed - type 22 - 600 x 1000 mm

- 1720 W@dT50

### Hallway

- Heat loss: 0.29 kW

- Design temperature: 21.0 oC

- Underfloor heating

- New - Low Profile Overlay

- Output 1274 W @ 27.0 oC floor temp

### Dining room

- Heat loss: 0.48 kW

- Design temperature: 21.0 oC

- Underfloor heating

- New - Low Profile Overlay

- Output 924 W @ 27.0 oC floor temp

### Wc

- Heat loss: 0.03 kW

- Design temperature: 21.0 oC

- Underfloor heating

- New - Low Profile Overlay

- Output 127 W @ 27.0 oC floor temp

### Lounge

- Heat loss: 0.93 kW

- Design temperature: 21.0 oC

- Underfloor heating

- New - Low Profile Overlay

- Output 1894 W @ 27.0 oC floor temp

### Kitchen

- Heat loss: 3.89 kW

- Design temperature: 20.0 oC

- Underfloor heating

- Existing - Screeded Floor

- Output 1928 W @ 27.2 oC floor temp

- Radiators

- Existing - type 21 - 500 x 1600 mm

- 1760 W@dT50

### Utility

- Heat loss: 0.25 kW

- Design temperature: 21.0 oC

### Garage

- Heat loss: 0.30 kW

- Design temperature: 15.0 oC

- Radiators

- Repurposed - type 22 - 600 x 1600 mm

- 2740 W@dT50

### Utility

- Heat loss: 0.25 kW

- Design temperature: 21.0 oC

### Garage

- Heat loss: 0.30 kW

- Design temperature: 15.0 oC

- Radiators

- Repurposed - type 22 - 600 x 1600 mm

- 2740 W@dT50

## First Floor

### Bedroom kids

- Heat loss: 0.23 kW

- Design temperature: 20.0 oC

- Radiators

- Existing - type 22 - 600 x 1600 mm

- 2740 W@dT50

### Landing

- Heat loss: 0.20 kW

- Design temperature: 20.0 oC

- Radiators

- Existing - type 22 - 600 x 1000 mm

- 1720 W@dT50

### Master bedroom

- Heat loss: 0.46 kW

- Design temperature: 20.0 oC

- Radiators

- Existing - type 22 - 600 x 1600 mm

- 2740 W@dT50

- Existing - type 22 - 450 x 1000 mm

- 1290 W@dT50

### Master en-suite

- Heat loss: 0.16 kW

- Design temperature: 20.0 oC

- Radiators

- Existing - type 10 - 1000 x 500 mm

- 590 W@dT50

### Bathroom

- Heat loss: 0.40 kW

- Design temperature: 20.0 oC

- Radiators

- Existing - type 21 - 600 x 1000 mm

- 1320 W@dT50

### Front bedroom 2

- Heat loss: 0.20 kW

- Design temperature: 20.0 oC

- Radiators

- Existing - type 22 - 600 x 1400 mm

- 2400 W@dT50

### Bedroom 3

- Heat loss: 0.57 kW

- Design temperature: 20.0 oC

- Radiators

- Existing - type 22 - 600 x 1600 mm

- 2740 W@dT50

### En-suite

- Heat loss: 0.11 kW

- Design temperature: 20.0 oC

- Radiators

- Existing - type 10 - 1000 x 500 mm 
590 W@dT50